735/3

GEOMETRICAL AND BUILDING DRAWING

Paper 3

AUGUST, 2017

3 HOURS

JINJA JOINT EXAMINATION BOARD

Uganda Certificate of Education

MOCK EXAMINATION – AUGUST,  2017

GEOMETRICAL AND BUILDING DRAWING

Paper 3

3 HOURS

INSTRUCTIONS TO CANDIDATES:

- This question paper consists of two sections I and II.

- Attempt ALL questions in this question paper.

- A drawing paper size A2 is provided, use both sides if necessary.

- Where dimensions are not given, use your discretion to determine missing dimensions.

- Figures in this paper are not drawn to scale.

- Interpret properly the specifications and figure provided before you attempt any question

Figure 1 provided below  shows a ground plan of A GUEST HOUSE not drawn to scale. The house is to be constructed using the following specifications:

SPECIFICATIONS:

ROOF:	Hipped roof pitched at 300 pitch angle covered with corrugated, 			galvanized iron sheets, on wooden purlines of 75mm x 50mm, on 			struts of 100mm x 50mm on rafters of 100mm x 50mm, on Tie beam 		150mm x 50mm, on wall plate of 100mm x 75mm finished with 			Fascia boards and verge boards of 200mm x 25mm.

WALLS:      Both internal and external walls are225mm thick and 					3000mm high from the finished floor to the underside of the wall 			plate built in ENGLISH BOND,

FOUNDATION: 	Concrete strip foundation of 750mm wide and 250mm thick, 				1200mm below the ground level.

FLOOR:		Finished with cement sand screed 25mm thick on site 					concrete 100mm thick, on hard core of 150mm thick, on well 				compacted marrum.

DOORS: 		D1 1500mm x 2100mm is double leaf metal casement with 				glass panes

D2 1000mm x 2100mm is a framed ledged braced battened 				door.

D3 900mm x 2100mm is a framed ledged braced and 					battened door.

WINDOWS: 	All windows are Louvered and fitted in wooden frames

W1 	2400 x 1500mm

W2 	900 x 1200mm

W3	900 x 1000m

SPLASH APRON	Concrete slab of 650 by 150mm

SECTION I

1. 	Using the Specifications given above, Draw a proportional free hand 	isometric sketch of the building with corner R as the fore point. 		           		                 							 (20 marks)

SECTION II

2	(a)	To a scale of 1:100, Draw:

(i)	the ground plan with all symbols indicated	(20 marks)

(ii) 	the cross-section S - S				(25 marks)

3.	To a scale 1: 10 draw the back elevation of D2 and name at least five parts  	      										(10 marks)

4.	At the bottom right corner of your drawing paper, print in the block your: 	Name, Index number, Title of the drawing, subject code and Date.(5 marks)